Lab Practical #02:

Study of basic networking commands and IP configuration.

Practical Assignment #02:

- Perform and explain various networking commands listed below:

  - traceroute / tracert

  - netstat

  - nslookup

  - pathping

  - arp

## traceroute/tracert

### Description:

The tracert command is used to trace the route that data packets take from your computer to a destination host and shows all the routers (hops) along the path.

| No. | Option | Description |
| --- | --- | --- |
| 1 | tracert | tracert asks, "What path does my data follow to reach a website or server?" and shows all the routers on the way. |
| 2 | tracert -4 | The tracert -4 command is used to force Tracert to use IPv4 addresses only when tracing the route to a destination. |
| 3 | tracert -h | tracert -h tells the computer, "Trace the route, but don't go beyond the specified number of routers (hops)." |
| 4 | tracert -d | tracert -d tells the computer, "Show only IP addresses while tracing the route, not the host names." |

### Implementation:

## netstat

### Description:

The netstat command is used to display active network connections, listening ports, and network statistics on a computer.

| No. | Option | Description |
| --- | --- | --- |
| 1 | netstat | netstat tells you which computers or websites your computer is currently connected to and which ports are being used. |
| 2 | netstat -a | The netstat -a command is used to display all active connections and listening ports on your computer. |
| 3 | netstat -f | netstat -f shows active network connections and displays website/server names instead of just IP addresses. |
| 4 | netstat -n | The netstat -n command is used to display active network connections using numerical IP addresses and port numbers only, without converting them to host names. |

### Implementation:

nslookup -type=A:

The nslookup -type=A command is used to find the IPv4 address (A Record) of a domain name.

nslookup -type=AAAA:

The nslookup -type=AAAA command is used to find the IPv6 address (AAAA Record) of a domain name.

Pathping:

The pathping command is used to trace the route to a destination and check network delays or packet loss at each hop.

pathping -h:

The pathping -h command is used to set the maximum number of hops (routers) to search while tracing the path to a destination.

pathping -n:

The pathping -n command is used to display IP addresses only while tracing the network path, without converting them to host names.

pathping -q:

The pathping -q command is used to set the number of query packets sent to each hop while testing the network path.

arp -a:

The arp -a command is used to display the ARP (Address Resolution Protocol) cache, which shows the mapping between IP addresses and MAC addresses on your local network.